Unit 2: Digital Logic — Complete Study Material

Welcome to Unit 2: Digital Logic of Introduction to I.T. Systems (DI01016021)! Digital logic is the foundation of computer architecture, hardware design, and digital computing systems. This material covers everything required for your syllabus: Number Systems, Boolean Algebra, Logic Gates, Combinational Circuits, and Sequential Circuits.

2.1 Number Systems & Code Conversions

Computers process and store all information in binary format. Understanding conversions between different bases is essential for understanding how hardware represents data.

1. Types of Number Systems

- Decimal System (Base 10): Uses digits 0 through 9.

- Binary System (Base 2): Uses digits 0 and 1.

- Octal System (Base 8): Uses digits 0 through 7.

- Hexadecimal System (Base 16): Uses digits 0–9 and letters A–F (where A=10,B=11,C=12,D=13,E=14,F=15).

2. Conversions Between Base Systems

A. Decimal to Binary / Octal / Hexadecimal

- Method: Successive division by the target base (2,8, or 16) and recording the remainders from bottom to top (Least Significant Bit to Most Significant Bit).

B. Binary / Octal / Hexadecimal to Decimal

- Method: Multiply each digit by its positional weight (power of the base) and sum the results.

Decimal Value=i=0n-1di×Basei

C. Binary to Hexadecimal / Octal Direct Conversion

- Binary to Octal: Group binary bits into sets of 3 bits from right to left.

- Binary to Hexadecimal: Group binary bits into sets of 4 bits from right to left.

3. Binary Arithmetic & Complements

1's and 2's Complement Representation

- 1's Complement: Invert all bits (0→1 and 1→0).

- 2's Complement: Add 1 to the 1's complement.

2's Complement=1's Complement+1

Note: 2's complement is widely used in modern computer hardware to represent signed negative integers and perform subtraction using addition circuitry.

2.2 Logic Gates & Truth Tables

Logic gates are the basic building blocks of any digital circuit. They take one or more binary inputs and produce a single binary output.

1. Basic Logic Gates

AND Gate             OR Gate            NOT Gate (Inverter)

A ───┐              A ───┐

│)─── Y             │)──── Y         A ───►|o─── Y

B ───┘              B ───┘

Y = A · B            Y = A + B               Y = A'

| Gate | Boolean Expression | Truth Table Summary |
| --- | --- | --- |
| AND | Y=A⋅B | Output is 1 only if all inputs are 1. |
| OR | Y=A+B | Output is 1 if at least one input is 1. |
| NOT | Y=A' or A | Output is the inverted value of the input. |

2. Universal Logic Gates

NAND and NOR gates are known as Universal Gates because any digital logic function (AND, OR, NOT) can be realized using only NAND or only NOR gates.

NAND Gate                           NOR Gate

A ───┐                             A ───┐

│)o─── Y = (A · B)'                │)o─── Y = (A + B)'

B ───┘                             B ───┘

- NAND Gate: Inverted AND gate output. Truth Table: Output is 0 only when both inputs are 1.

- NOR Gate: Inverted OR gate output. Truth Table: Output is 1 only when both inputs are 0.

3. Exclusive Gates (XOR & XNOR)

- XOR Gate (Exclusive-OR): Y=A⊕B=A'B+AB'.

  - Rule: Output is 1 when the inputs are different.

- XNOR Gate (Exclusive-NOR): Y=A⊕B'=AB+A'B'.

  - Rule: Output is 1 when the inputs are identical.

2.3 Laws of Boolean Algebra & Canonical Forms

Boolean algebra is used to simplify complex logical expressions, reducing the number of logic gates needed in physical hardware.

Key Boolean Laws

| Law Name | Mathematical Expression |
| --- | --- |
| Identity Laws | A⋅1=A ∣ A+0=A |
| Null / Dominance Laws | A⋅0=0 ∣ A+1=1 |
| Idempotent Laws | A⋅A=A ∣ A+A=A |
| Complement Laws | A⋅A'=0 ∣ A+A'=1 |
| Commutative Laws | A⋅B=B⋅A ∣ A+B=B+A |
| Associative Laws | ABC=ABC ∣ A+B+C=A+B+C |
| Distributive Laws | AB+C=AB+AC ∣ A+BC=A+BA+C |
| De Morgan's Theorems | Theorem 1: A⋅B'=A'+B' Theorem 2: A+B'=A'⋅B' |

Canonical Logic Forms: SOP and POS

- Sum of Products (SOP): Formed by combining Minterms (m) with OR operators (e.g., F=A'B+AB').

- Product of Sums (POS): Formed by combining Maxterms (M) with AND operators (e.g., F=A+BA'+B').

2.4 Combinational Logic Circuits

A Combinational Circuit is a digital circuit where the output at any instant depends only on the current input values. It has no internal memory elements.

┌───────────────────────────────┐

Inputs (A, B) ─┤  Combinational Logic Circuit  ├─ Outputs (Sum, Carry)

└───────────────────────────────┘

1. Adders (Half Adder & Full Adder)

- Half Adder: Adds two 1-bit binary numbers (A and B).

  - Sum S=A⊕B

  - Carry C=A⋅B

- Full Adder: Adds three 1-bit binary numbers (A, B, and input carry Cin).

  - Sum S=A⊕B⊕Cin

  - Carry Cout=AB+BCin+ACin

2. Data Selectors & Distributors

- Multiplexer (MUX / Data Selector): Selects one digital signal from multiple input lines and routes it to a single output line based on selection lines.

  - Configuration: 2n inputs to 1 output with n select lines (e.g., 4-to-1 MUX has 4 inputs, 1 output, and 2 select lines).

- Demultiplexer (DEMUX / Data Distributor): Takes a single input line and routes it to one of many output lines.

3. Encoders and Decoders

- Encoder: Converts an active input signal into a coded binary output (e.g., Octal-to-Binary / 8-to-3 Encoder).

- Decoder: Converts coded binary inputs into a single active output line (e.g., 2-to-4 Decoder, 3-to-8 Decoder, BCD to 7-Segment Decoder).

2.5 Basics of Sequential Circuits

A Sequential Circuit is a circuit whose output depends not only on current inputs but also on past outputs (stored state).

Sequential Circuit=Combinational Logic+Memory Elements (Flip-Flops)

┌───────────────────────────────┐

Inputs ──────►│  Combinational Logic Circuit  ├─────────► Outputs

└───────────────▲───────────────┘

│

┌───────┴───────┐

│ Memory Elements│

│ (Flip-Flops)  │

└───────────────┘

Key Flip-Flops (1-Bit Storage Elements)

- SR Flip-Flop (Set-Reset): Basic storage cell; input state S=1,R=1 is invalid/undefined.

- D Flip-Flop (Data/Delay): Eliminates the invalid state; stores whatever bit is applied at input D on clock trigger.

- JK Flip-Flop: Resolves SR undefined condition; when J=1,K=1, the output toggles.

- T Flip-Flop (Toggle): Toggles its output state on every clock pulse when T=1.

Step-by-Step Solved Examples

Solved Example 1: Number System Conversion

Problem: Convert Decimal 2510 to Binary.

- Solution:

  - 25÷2=12 Remainder 1 LSB

  - 12÷2=6 Remainder 0

  - 6÷2=3 Remainder 0

  - 3÷2=1 Remainder 1

  - 1÷2=0 Remainder 1 MSB

  - Read remainders from bottom to top: 2510=110012.

Solved Example 2: Simplifying a Boolean Expression

Problem: Simplify the Boolean expression F=A'B+AB+AB'.

- Solution:

  - Group the first two terms: F=BA'+A+AB'

  - Apply Complement Law (A'+A=1): F=B1+AB'

  - Simplify: F=B+AB'

  - Apply Distributive Law (B+AB'=B+AB+B'):

    - Since B+B'=1, F=A+B.

  - Final Simplified Expression: F=A+B (OR Gate).

Unit 2 Practice Exercise

- Convert the binary number 11011012 into both Octal and Hexadecimal representations.

- Find the 1's complement and 2's complement of 101011002.

- Prove De Morgan's First Theorem A⋅B'=A'+B' using a truth table.

- Draw the truth table and write the logic expressions for the Sum and Carry outputs of a Half Adder.

- Differentiate between Combinational Circuits and Sequential Circuits with two examples each.