Unit 4: Computer Networks & Security — Complete Study Material

Welcome to Unit 4: Computer Networks & Security of Introduction to I.T. Systems (DI01016021)! Computer networking enables interconnected computers to share resources, exchange data, and communicate seamlessly across the globe. This unit covers network fundamentals, reference models, hardware devices, internet protocols, and core cybersecurity principles.

4.1 Fundamentals of Computer Networks

A Computer Network is a collection of interconnected computing devices (nodes) that communicate using shared protocols over physical or wireless transmission media.

1. Classification by Geographical Scale

| Network Type | Full Form | Coverage Area | Typical Use Case |
| --- | --- | --- | --- |
| PAN | Personal Area Network | Up to 10 meters | Connecting a smartphone, smartwatch, and Bluetooth headphones. |
| LAN | Local Area Network | Within a room, office, or building (1 km) | Interconnecting lab PCs or office computers. |
| MAN | Metropolitan Area Network | Across a city (10 to 50 km) | Cable TV networks or university campuses spread across a town. |
| WAN | Wide Area Network | Country, continent, or global scale | The Internet; connecting bank branch offices globally. |

2. Network Topologies

Network topology refers to the geometric or logical arrangement of computing devices and links in a network.

- Bus Topology: All nodes connect to a single central backbone cable.

  - Advantage: Easy to set up and requires minimal cabling.

  - Disadvantage: If the central cable breaks, the entire network fails.

- Star Topology: Every node connects independently to a central hub or switch.

  - Advantage: Single node failure does not affect others; easy to troubleshoot.

  - Disadvantage: Failure of the central switch disables the whole network.

- Ring Topology: Nodes are connected sequentially in a closed loop; data travels in one direction.

  - Advantage: No data collisions due to token passing.

  - Disadvantage: Failure of a single workstation disrupts the whole ring.

- Mesh Topology: Every node is connected directly to every other node (Full Mesh).

  - Advantage: High redundancy, fault tolerance, and privacy.

  - Formula for links: NN-1/2 dedicated links for N nodes.

  - Disadvantage: High cost and complex installation due to massive cabling.

4.2 Network Reference Models

Network reference models standardise the communication process across different hardware and software vendors.

1. The OSI 7-Layer Reference Model

The Open Systems Interconnection (OSI) model is a conceptual framework designed by ISO to standardize network communication into seven distinct layers.

- 7. Application Layer: Interfaces directly with user applications (HTTP, FTP, SMTP, DNS).

- 6. Presentation Layer: Handles data translation, encryption, decryption, and compression (SSL/TLS, JPEG, ASCII).

- 5. Session Layer: Establishes, manages, and terminates communication sessions between applications.

- 4. Transport Layer: Provides end-to-end reliability, flow control, error correction, and port addressing (TCP, UDP). Data unit: Segments.

- 3. Network Layer: Manages logical IP addressing and packet routing across networks (IP, ICMP). Data unit: Packets.

- 2. Data Link Layer: Handles physical MAC addressing, framing, and node-to-node error detection (Ethernet, Wi-Fi). Data unit: Frames.

- 1. Physical Layer: Transmits raw bitstreams over physical copper wires, fiber optics, or radio waves. Data unit: Bits.

2. OSI Model vs. TCP/IP Model

The TCP/IP Model is the practical architecture used on the modern Internet.

OSI Model (7 Layers)               TCP/IP Model (4 Layers)

┌────────────────────────────────┐    ┌────────────────────────────────┐

│ 7. Application                 │    │                                │

├────────────────────────────────┤    │ 4. Application Layer           │

│ 6. Presentation                │    │    (HTTP, FTP, DNS, SSH)       │

├────────────────────────────────┤    │                                │

│ 5. Session                     │    │                                │

├────────────────────────────────┤    ├────────────────────────────────┤

│ 4. Transport                   │───►│ 3. Transport Layer (TCP, UDP)  │

├────────────────────────────────┤    ├────────────────────────────────┤

│ 3. Network                     │───►│ 2. Internet Layer (IP, ICMP)   │

├────────────────────────────────┤    ├────────────────────────────────┤

│ 2. Data Link                   │───►│ 1. Network Access Layer        │

├────────────────────────────────┤    │    (Ethernet, Wi-Fi)           │

│ 1. Physical                    │    │                                │

└────────────────────────────────┘    └────────────────────────────────┘

4.3 Network Hardware Devices & Protocols

Key Network Devices

- Hub: A Layer-1 physical device that broadcasts incoming data frames to all connected ports (high collision rate).

- Switch: A Layer-2 data link device that uses MAC address tables to intelligently forward frames only to the intended destination device.

- Router: A Layer-3 network device that inspects IP addresses to route data packets between different subnets and networks.

- Modem (Modulator-Demodulator): Converts digital signals from a computer into analog signals for transmission over telecom lines and vice versa.

Core Internet Protocols

- IP (Internet Protocol): Assigns logical addresses to network interfaces; routes packets across networks (IPv4 is 32-bit, IPv6 is 128-bit).

- TCP vs UDP:

  - TCP (Transmission Control Protocol): Connection-oriented, reliable, guarantees packet arrival and correct ordering (used in web browsing, email).

  - UDP (User Datagram Protocol): Connectionless, fast, lightweight with no arrival guarantees (used in live video streaming, online gaming).

- DNS (Domain Name System): Translates human-readable domain names (e.g., [www.google.com](https://www.google.com)) into numerical IP addresses (e.g., 142.250.190.46).

- DHCP (Dynamic Host Configuration Protocol): Automatically assigns IP addresses, subnet masks, and default gateways to devices joining a network.

4.4 Fundamentals of Cybersecurity

Cybersecurity involves protecting computer systems, networks, and data from digital attacks, unauthorized access, and damage.

1. The CIA Triad

The CIA Triad forms the foundational framework for information security policies:

┌───────────────────┐

│   Confidentiality │

└─────────┬─────────┘

│

┌────────────┴────────────┐

│                         │

▼                         ▼

┌────────────────────┐     ┌───────────────────┐

│     Integrity      │     │   Availability    │

└────────────────────┘     └───────────────────┘

- Confidentiality: Ensures sensitive data is accessible only to authorized users (achieved via Encryption, Passwords, RBAC).

- Integrity: Guarantees that data is accurate, complete, and protected against unauthorized modification or tampering (achieved via Hashing, Digital Signatures).

- Availability: Ensures systems, services, and data are consistently accessible to authorized users when needed (achieved via Redundancy, Backups, DDoS Protection).

2. Common Cyber Threats & Attacks

- Malware (Malicious Software):

  - Virus: Attaches to legitimate files and spreads when the file is executed.

  - Worm: Self-replicating program that spreads across networks without human intervention.

  - Trojan Horse: Disguises itself as legitimate software to trick users into running it.

  - Ransomware: Encrypts victim files and demands payment for the decryption key.

- Phishing: Deceptive communication (emails, websites) designed to trick users into revealing sensitive credentials or personal details.

- Man-in-the-Middle (MitM): An attacker secretly intercepts and relays communication between two parties who believe they are directly communicating.

- Denial of Service (DoS / DDoS): Floods a target server or network with excessive traffic to crash it and deny service to legitimate users.

3. Basic Security Safeguards

- Firewall: A network security system that monitors and filters incoming and outgoing network traffic based on predefined security rules.

- Encryption:

  - Symmetric Encryption: Uses the same secret key for both encryption and decryption (e.g., AES).

  - Asymmetric Encryption: Uses a public key for encryption and a matching private key for decryption (e.g., RSA).

- Multi-Factor Authentication (MFA): Requires users to present two or more independent credentials (e.g., Password + OTP code) before granting system access.

Step-by-Step Solved Example: Network Link Calculation

Problem: Mesh Topology Cables

An IT office building needs to connect 6 server machines using a Full Mesh Topology. Calculate the total number of dedicated physical links required.

Solution

- Formula for Full Mesh Links:

Links=N×N-12

where N is the total number of nodes in the network.

- Substitute N=6:

Links=6×6-12

Links=6×52=302=15

- Final Answer: A total of 15 dedicated cable links are required to build a full mesh topology for 6 servers.

Unit 4 Practice Exercise

- Compare LAN, MAN, and WAN on the basis of coverage area, data transfer rate, and ownership.

- List all 7 layers of the OSI Reference Model from Layer 1 to Layer 7 and state one primary function for each.

- Differentiate between a Switch and a Router with respect to OSI operational layer and primary purpose.

- Explain the CIA Triad in cybersecurity with a real-world banking example.

- Define Phishing and Ransomware, and describe two methods to protect against them.